Practical – 1

Aim: Installation: ns3 in Linux, NetAnim, Wireshark, PyViz, tcpdump

Steps:

sudo apt upgradesudo apt update

sudo apt-get install g++ pythonsudo apt-get install g++ python3 python3-dev pkg-config sqlite3

sudo apt install -y qtcreator qtbase5-dev qt5-qmake cmake

sudo apt-get install gir1.2-goocanvas-2.0 python3-pygraphviz python3-gi python3-gi-cairo gir1.2-gtk-3.0

sudo apt-get install gdb valgrindsudo apt-get install doxygen graphviz imagemagick

sudo apt install python3-pipsudo apt-get install python3-sphinx dia

sudo apt-get install tcpdumpsudo apt-get install -y llvm-11 llvm-11-dev clang-11 llvm-11-tools

pip install cxxfiltwget https://www.nsnam.org/releases/ns-allinone-3.32.tar.bz2

tar -xjf ns-allinone-3.32.tar.bz2

cd ns-allinone-3.39/ns-3.39

./waf configure --enable-examples --enable-tests

./waf build

./waf --run "scratch/scratch-simulator"

cd ~/ns-allinone-3.32/netanim-3.108

sudo apt install -y qt5-default qttools5-dev qttools5-dev-tools

./waf build

sudo apt-get update

sudo apt-get install qtbase5-dev qtchooser qt5-qmake qtbase5-dev-tools build-essential

qmake NetAnim.pro

make

./NetAnim

sudo apt install wireshark

sudo usermod -aG wireshark $(whoami)

sudo reboot

sudo apt update

sudo apt install tcpdump -y

tcpdump –version

sudo tcpdump -i any -w capture.pcap

wireshark capture.pcap

Installation of pyviz visualizeer

➢ Install python3-venv

• sudo apt update

• sudo apt install python3-venv

python3 -m venv pyviz-env

pip install holoviews panel datashader bokeh hvplot

pip install jupyterlab notebook ipywidgets

>>> import holoviews as hv

>>> hv.extension('bokeh')

>>> hv.Curve([1, 2, 3])

Practical - 2

Aim: Program to simulate Point to Point topology.

point77.cc

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/netanim-module.h"

#include "ns3/mobility-module.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("FirstScriptExample");

int main(int argc, char *argv[])

{

// Parse command line arguments

CommandLine cmd;

cmd.Parse(argc, argv);

// Set the resolution of time (nanoseconds)

Time::SetResolution(Time::NS);

// Enable logging for the UDP Echo client and server applications

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

// Create two nodes (n0 and n1)

NodeContainer nodes;

nodes.Create(2);

// Set up the Point-to-Point link between the two nodes

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

// Install the Point-to-Point devices on the nodes

NetDeviceContainer devices;

devices = pointToPoint.Install(nodes);

// Install the Internet stack (IP, UDP, etc.) on the nodes

InternetStackHelper stack;

stack.Install(nodes);

// Set IP addresses for the devices

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer interfaces = address.Assign(devices);

// Set up the UDP Echo server on node 1, listening on port 9

UdpEchoServerHelper echoServer(15);

ApplicationContainer serverApps = echoServer.Install(nodes.Get(1));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

// Set up the UDP Echo client on node 0, sending to node 1 at port 9

UdpEchoClientHelper echoClient(interfaces.GetAddress(1), 15);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

// Install the UDP Echo client application on node 0

ApplicationContainer clientApps = echoClient.Install(nodes.Get(0));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

// Set the mobility model for both nodes (Constant Position)

MobilityHelper mobility;

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel");

mobility.Install(nodes);

// Set specific positions for the nodes (n0 at (10,25), n1 at (40,25))

AnimationInterface anim("first.xml");

AnimationInterface::SetConstantPosition(nodes.Get(0), 10, 25);

AnimationInterface::SetConstantPosition(nodes.Get(1), 40, 25);

// Enable packet metadata in NetAnim (optional but useful for debugging)

anim.EnablePacketMetadata(true);

// Enable Pcap trace for packet capture

pointToPoint.EnablePcapAll("point");

// Run the simulation

Simulator::Run();

Simulator::Destroy();

return 0;

}

OUTPUT:

Change the Port Number (15 to 16):

Python Visualization:

2. Point to point Topology using NetAnim

first77.cc

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/netanim-module.h"

#include "ns3/mobility-module.h"

// Default Network Topology

// 10.1.1.0

// n0 --- n1

// Point-to-Point link

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("FirstScriptExample");

int main(int argc, char *argv[])

{

CommandLine cmd;

cmd.Parse(argc, argv);

// Set time resolution to nanoseconds

Time::SetResolution(Time::NS);

// Enable logging for UDP Echo Client and Server

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

// Create 2 nodes

NodeContainer nodes;

nodes.Create(2);

// Configure point-to-point connection

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

// Install devices

NetDeviceContainer devices;

devices = pointToPoint.Install(nodes);

InternetStackHelper stack;

stack.Install(nodes);

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer interfaces = address.Assign(devices);

// Create and configure UDP Echo Server on node 1 (n1)

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps = echoServer.Install(nodes.Get(1));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

// Create and configure UDP Echo Client on node 0 (n0)

UdpEchoClientHelper echoClient(interfaces.GetAddress(1), 9);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps = echoClient.Install(nodes.Get(0));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

// Set constant mobility for animation

MobilityHelper mobility;

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel");

mobility.Install(nodes);

// Configure NetAnim

AnimationInterface anim("first.xml");

AnimationInterface::SetConstantPosition(nodes.Get(0), 10, 25);

AnimationInterface::SetConstantPosition(nodes.Get(1), 40, 25);

anim.EnablePacketMetadata(true);

// Enable PCAP tracing

pointToPoint.EnablePcapAll("pract2Mana");

// Run the simulation

Simulator::Run();

Simulator::Destroy();

return 0;

}

OUTPUT:

• Open NetAnim to see the animation:

• Run the first.xml file for visualization:

Practical – 3

Aim: Program to simulate Bus topology

bus77.cc

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/ipv4-global-routing-helper.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("SecondScriptExample");

int main(int argc, char *argv[])

{

bool verbose = true;

uint32_t nCsma = 5;

// Command line argument parsing

CommandLine cmd;

cmd.AddValue("nCsma", "Number of CSMA nodes/devices", nCsma);

cmd.AddValue("verbose", "Enable logging if true", verbose);

cmd.Parse(argc, argv);

// Enable logging

if (verbose)

{

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

// Ensure at least 1 CSMA node

if (nCsma == 0)

{

nCsma = 1;

}

// Create point-to-point nodes

NodeContainer p2pNodes;

p2pNodes.Create(2);

// Create CSMA nodes and connect one P2P node

NodeContainer csmaNodes;

csmaNodes.Add(p2pNodes.Get(1));

csmaNodes.Create(nCsma);

// Configure point-to-point link

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("10Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("5ms"));

NetDeviceContainer p2pDevices;

p2pDevices = pointToPoint.Install(p2pNodes);

// Configure CSMA network

CsmaHelper csma;

csma.SetChannelAttribute("DataRate", StringValue("100Mbps"));

csma.SetChannelAttribute("Delay", TimeValue(NanoSeconds(6560)));

NetDeviceContainer csmaDevices;

csmaDevices = csma.Install(csmaNodes);

// Install Internet stack

InternetStackHelper stack;

stack.Install(p2pNodes.Get(0));

stack.Install(csmaNodes);

// Assign IP addresses (P2P)

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign(p2pDevices);

// Assign IP addresses (CSMA)

address.SetBase("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces;

csmaInterfaces = address.Assign(csmaDevices);

// Create UDP Echo Server (last CSMA node)

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps;

serverApps = echoServer.Install(csmaNodes.Get(nCsma));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

// Create UDP Echo Client (P2P node)

UdpEchoClientHelper echoClient(csmaInterfaces.GetAddress(nCsma), 9);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps;

clientApps = echoClient.Install(p2pNodes.Get(0));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

// Populate routing tables

Ipv4GlobalRoutingHelper::PopulateRoutingTables();

// Enable PCAP tracing

pointToPoint.EnablePcapAll("pract2Mana");

csma.EnablePcap("pract2Mana", csmaDevices.Get(1), true);

// Run simulation

Simulator::Run();

Simulator::Destroy();

return 0;

}

OUTPUT:

Practical – 4

Aim: Program to simulate Start topology

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-star.h"

#include "ns3/applications-module.h"

#include "ns3/netanim-module.h"

#include "ns3/mobility-module.h"

#include "ns3/onoff-application.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("StarExample");

int main(int argc, char *argv[])

{

// Set default packet size and data rate

Config::SetDefault("ns3::OnOffApplication::PacketSize", UintegerValue(137));

Config::SetDefault("ns3::OnOffApplication::DataRate", StringValue("14kb/s"));

// Default number of spokes

uint32_t nSpokes = 8;

// Command line argument

CommandLine cmd;

cmd.AddValue("nSpokes", "Number of nodes in the star", nSpokes);

cmd.Parse(argc, argv);

// Enable logging

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

NS_LOG_INFO("Build star topology");

// Configure point-to-point links

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

// Create star topology

PointToPointStarHelper star(nSpokes, pointToPoint);

NS_LOG_INFO("Install Internet Stack");

InternetStackHelper internet;

star.InstallStack(internet);

NS_LOG_INFO("Assign IP Addresses");

star.AssignIpv4Addresses(Ipv4AddressHelper("10.1.1.0", "255.255.255.0"));

NS_LOG_INFO("Create Applications");

// Create Packet Sink (Server) on Hub

uint16_t port = 50000;

Address hubLocalAddress(InetSocketAddress(Ipv4Address::GetAny(), port));

PacketSinkHelper packetSinkHelper("ns3::TcpSocketFactory", hubLocalAddress);

ApplicationContainer hubApp = packetSinkHelper.Install(star.GetHub());

hubApp.Start(Seconds(1.0));

hubApp.Stop(Seconds(10.0));

// Create OnOff Applications on each spoke

OnOffHelper onOffHelper("ns3::TcpSocketFactory", Address());

onOffHelper.SetAttribute("OnTime",

StringValue("ns3::ConstantRandomVariable[Constant=1]"));

onOffHelper.SetAttribute("OffTime",

StringValue("ns3::ConstantRandomVariable[Constant=0]"));

ApplicationContainer spokeApps;

for (uint32_t i = 0; i < star.SpokeCount(); ++i)

{

AddressValue remoteAddress(

InetSocketAddress(star.GetHubIpv4Address(i), port));

onOffHelper.SetAttribute("Remote", remoteAddress);

spokeApps.Add(onOffHelper.Install(star.GetSpokeNode(i)));

}

spokeApps.Start(Seconds(1.0));

spokeApps.Stop(Seconds(10.0));

NS_LOG_INFO("Enable Routing");

Ipv4GlobalRoutingHelper::PopulateRoutingTables();

NS_LOG_INFO("Enable PCAP tracing");

pointToPoint.EnablePcapAll("star");

// Set layout for NetAnim

star.BoundingBox(1, 1, 100, 100);

// NetAnim output file

AnimationInterface anim("star.xml");

NS_LOG_INFO("Run Simulation");

Simulator::Run();

Simulator::Destroy();

NS_LOG_INFO("Done");

return 0;

}

Using Python Visualizer :

Using NetAnim:

Practical – 5

Aim: Program to simulate Mesh topology

#include "ns3/applications-module.h"

#include "ns3/core-module.h"

#include "ns3/internet-module.h"

#include "ns3/mesh-helper.h"

#include "ns3/mesh-module.h"

#include "ns3/mobility-module.h"

#include "ns3/network-module.h"

#include "ns3/yans-wifi-helper.h"

#include "ns3/netanim-module.h"

#include <fstream>

#include <iostream>

#include <sstream>

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("MeshExample");

// Global counters

uint32_t g_udpTxCount = 0;

uint32_t g_udpRxCount = 0;

// TX Trace

void TxTrace(Ptr<const Packet> p)

{

NS_LOG_DEBUG("Sent " << p->GetSize() << " bytes");

g_udpTxCount++;

}

// RX Trace

void RxTrace(Ptr<const Packet> p)

{

NS_LOG_DEBUG("Received " << p->GetSize() << " bytes");

g_udpRxCount++;

}

class MeshTest

{

public:

MeshTest();

void Configure(int argc, char **argv);

int Run();

private:

// Parameters

int m_xSize;

int m_ySize;

double m_step;

double m_randomStart;

double m_totalTime;

double m_packetInterval;

uint16_t m_packetSize;

uint32_t m_nIfaces;

bool m_chan;

bool m_pcap;

bool m_ascii;

std::string m_stack;

std::string m_root;

// Containers

NodeContainer nodes;

NetDeviceContainer meshDevices;

Ipv4InterfaceContainer interfaces;

MeshHelper mesh;

// Functions

void CreateNodes();

void InstallInternetStack();

void InstallApplication();

void Report();

};

// ================= CONSTRUCTOR =================

MeshTest::MeshTest()

: m_xSize(3),

m_ySize(3),

m_step(50.0),

m_randomStart(0.1),

m_totalTime(100.0),

m_packetInterval(1.0),

m_packetSize(1024),

m_nIfaces(1),

m_chan(true),

m_pcap(false),

m_ascii(false),

m_stack("ns3::Dot11sStack"),

m_root("ff:ff:ff:ff:ff:ff")

{

}

void MeshTest::Configure(int argc, char *argv[])

{

CommandLine cmd;

cmd.AddValue("x-size", "Grid width", m_xSize);

cmd.AddValue("y-size", "Grid height", m_ySize);

cmd.AddValue("step", "Distance between nodes", m_step);

cmd.AddValue("start", "Random start delay", m_randomStart);

cmd.AddValue("time", "Simulation time", m_totalTime);

cmd.AddValue("packet-interval", "Packet interval", m_packetInterval);

cmd.AddValue("packet-size", "Packet size", m_packetSize);

cmd.AddValue("interfaces", "Number of interfaces", m_nIfaces);

cmd.AddValue("channels", "Use multiple channels", m_chan);

cmd.AddValue("pcap", "Enable PCAP", m_pcap);

cmd.AddValue("ascii", "Enable ASCII", m_ascii);

cmd.AddValue("stack", "Protocol stack", m_stack);

cmd.AddValue("root", "Root MAC", m_root);

cmd.Parse(argc, argv);

if (m_ascii)

{

PacketMetadata::Enable();

}

}

// ================= CREATE NODES =================

void MeshTest::CreateNodes()

{

nodes.Create(m_xSize * m_ySize);

YansWifiPhyHelper wifiPhy = YansWifiPhyHelper::Default();

YansWifiChannelHelper wifiChannel = YansWifiChannelHelper::Default();

wifiPhy.SetChannel(wifiChannel.Create());

mesh = MeshHelper::Default();

if (!Mac48Address(m_root.c_str()).IsBroadcast())

{

mesh.SetStackInstaller(m_stack, "Root",

Mac48AddressValue(Mac48Address(m_root.c_str())));

}

else

{

mesh.SetStackInstaller(m_stack);

}

mesh.SetSpreadInterfaceChannels(

m_chan ? MeshHelper::SPREAD_CHANNELS : MeshHelper::ZERO_CHANNEL);

mesh.SetMacType("RandomStart", TimeValue(Seconds(m_randomStart)));

mesh.SetNumberOfInterfaces(m_nIfaces);

meshDevices = mesh.Install(wifiPhy, nodes);

// Mobility

MobilityHelper mobility;

mobility.SetPositionAllocator("ns3::GridPositionAllocator",

"MinX", DoubleValue(0.0),

"MinY", DoubleValue(0.0),

"DeltaX", DoubleValue(m_step),

"DeltaY", DoubleValue(m_step),

"GridWidth", UintegerValue(m_xSize),

"LayoutType", StringValue("RowFirst"));

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel");

mobility.Install(nodes);

if (m_pcap)

{

wifiPhy.EnablePcapAll("mesh");

}

}

void MeshTest::InstallInternetStack()

{

InternetStackHelper internet;

internet.Install(nodes);

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

interfaces = address.Assign(meshDevices);

}

// ================= APPLICATION =================

void MeshTest::InstallApplication()

{

uint16_t port = 9;

uint32_t sinkNode = m_xSize * m_ySize - 1;

UdpEchoServerHelper server(port);

ApplicationContainer serverApp = server.Install(nodes.Get(sinkNode));

serverApp.Start(Seconds(1.0));

serverApp.Stop(Seconds(m_totalTime + 1));

UdpEchoClientHelper client(interfaces.GetAddress(sinkNode), port);

client.SetAttribute("MaxPackets",

UintegerValue((uint32_t)(m_totalTime / m_packetInterval)));

client.SetAttribute("Interval",

TimeValue(Seconds(m_packetInterval)));

client.SetAttribute("PacketSize",

UintegerValue(m_packetSize));

ApplicationContainer clientApp = client.Install(nodes.Get(0));

Ptr<UdpEchoClient> app = clientApp.Get(0)->GetObject<UdpEchoClient>();

app->TraceConnectWithoutContext("Tx", MakeCallback(&TxTrace));

app->TraceConnectWithoutContext("Rx", MakeCallback(&RxTrace));

clientApp.Start(Seconds(1.0));

clientApp.Stop(Seconds(m_totalTime + 1.5));

}

int MeshTest::Run()

{

CreateNodes();

InstallInternetStack();

InstallApplication();

AnimationInterface anim("mesh-topology.xml");

Simulator::Schedule(Seconds(m_totalTime), &MeshTest::Report, this);

Simulator::Stop(Seconds(m_totalTime + 2));

Simulator::Run();

Simulator::Destroy();

std::cout << "Packets Sent: " << g_udpTxCount

<< " Received: " << g_udpRxCount << std::endl;

return 0;

}

void MeshTest::Report()

{

unsigned n = 0;

for (auto i = meshDevices.Begin(); i != meshDevices.End(); ++i, ++n)

{

std::ostringstream os;

os << "mp-report-" << n << ".xml";

std::ofstream of(os.str());

mesh.Report(*i, of);

of.close();

}

}

int main(int argc, char *argv[])

{

MeshTest t;

t.Configure(argc, argv);

return t.Run();

}

Output:

Using python Visualizer:

Practical - 6

Aim: Program to simulate Hybrid topology

#include "ns3/core-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/network-module.h"

#include "ns3/applications-module.h"

#include "ns3/mobility-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

#include "ns3/yans-wifi-helper.h"

#include "ns3/ssid.h"

#include "ns3/netanim-module.h"

#include "ns3/flow-monitor-helper.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("ThirdScriptExample");

int main(int argc, char *argv[])

{

bool verbose = true;

uint32_t nCsma = 3;

uint32_t nWifi = 3;

bool tracing = false;

CommandLine cmd(__FILE__);

cmd.AddValue("nCsma", "Number of CSMA nodes", nCsma);

cmd.AddValue("nWifi", "Number of WiFi STA nodes", nWifi);

cmd.AddValue("verbose", "Enable logging", verbose);

cmd.AddValue("tracing", "Enable PCAP tracing", tracing);

cmd.Parse(argc, argv);

if (nWifi > 18)

{

std::cout << "nWifi should be <= 18 (layout limit)" << std::endl;

return 1;

}

if (verbose)

{

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

// ================= P2P SETUP =================

NodeContainer p2pNodes;

p2pNodes.Create(2);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

NetDeviceContainer p2pDevices = pointToPoint.Install(p2pNodes);

// ================= CSMA SETUP =================

NodeContainer csmaNodes;

csmaNodes.Add(p2pNodes.Get(1));

csmaNodes.Create(nCsma);

CsmaHelper csma;

csma.SetChannelAttribute("DataRate", StringValue("100Mbps"));

csma.SetChannelAttribute("Delay", TimeValue(NanoSeconds(6560)));

NetDeviceContainer csmaDevices = csma.Install(csmaNodes);

// ================= WIFI SETUP =================

NodeContainer wifiStaNodes;

wifiStaNodes.Create(nWifi);

NodeContainer wifiApNode = p2pNodes.Get(0);

YansWifiChannelHelper channel = YansWifiChannelHelper::Default();

YansWifiPhyHelper phy = YansWifiPhyHelper::Default();

phy.SetChannel(channel.Create());

WifiHelper wifi;

wifi.SetRemoteStationManager("ns3::AarfWifiManager");

WifiMacHelper mac;

Ssid ssid = Ssid("ns-3-ssid");

// STA

mac.SetType("ns3::StaWifiMac",

"Ssid", SsidValue(ssid),

"ActiveProbing", BooleanValue(false));

NetDeviceContainer staDevices = wifi.Install(phy, mac, wifiStaNodes);

// AP

mac.SetType("ns3::ApWifiMac",

"Ssid", SsidValue(ssid));

NetDeviceContainer apDevices = wifi.Install(phy, mac, wifiApNode);

// ================= MOBILITY =================

MobilityHelper mobility;

mobility.SetPositionAllocator("ns3::GridPositionAllocator",

"MinX", DoubleValue(0.0),

"MinY", DoubleValue(0.0),

"DeltaX", DoubleValue(5.0),

"DeltaY", DoubleValue(10.0),

"GridWidth", UintegerValue(3),

"LayoutType", StringValue("RowFirst"));

mobility.SetMobilityModel("ns3::RandomWalk2dMobilityModel",

"Bounds", RectangleValue(Rectangle(-50, 50, -50, 50)));

mobility.Install(wifiStaNodes);

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel");

mobility.Install(wifiApNode);

// ================= INTERNET =================

InternetStackHelper stack;

stack.Install(csmaNodes);

stack.Install(wifiApNode);

stack.Install(wifiStaNodes);

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces = address.Assign(p2pDevices);

address.SetBase("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces = address.Assign(csmaDevices);

address.SetBase("10.1.3.0", "255.255.255.0");

address.Assign(staDevices);

address.Assign(apDevices);

// ================= APPLICATION =================

UdpEchoServerHelper server(9);

ApplicationContainer serverApps = server.Install(csmaNodes.Get(nCsma));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

UdpEchoClientHelper client(csmaInterfaces.GetAddress(nCsma), 9);

client.SetAttribute("MaxPackets", UintegerValue(1));

client.SetAttribute("Interval", TimeValue(Seconds(1.0)));

client.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps =

client.Install(wifiStaNodes.Get(nWifi - 1));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

Ipv4GlobalRoutingHelper::PopulateRoutingTables();

// ================= NETANIM =================

AnimationInterface anim("third-animation.xml");

anim.SetConstantPosition(p2pNodes.Get(0), 1.0, 2.0);

anim.SetConstantPosition(p2pNodes.Get(1), 2.0, 2.0);

// ================= FLOW MONITOR =================

FlowMonitorHelper flowmon;

Ptr<FlowMonitor> monitor = flowmon.InstallAll();

// ================= TRACING =================

if (tracing)

{

pointToPoint.EnablePcapAll("third");

phy.EnablePcap("third", apDevices.Get(0));

csma.EnablePcap("third", csmaDevices.Get(0), true);

}

// ================= RUN =================

Simulator::Stop(Seconds(10.0));

Simulator::Run();

monitor->SerializeToXmlFile("third-flowmon.xml", true, true);

Simulator::Destroy();

return 0;

}

OUTPUT:

Using Python Visualizer

Open NetAnim to see the animation:

Practical – 7

Aim: Program to simulate UDP Client Server

udp77.cc

#include <fstream>

#include "ns3/core-module.h"

#include "ns3/csma-module.h"

#include "ns3/applications-module.h"

#include "ns3/internet-module.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("UdpClientServerExample");

int main(int argc, char *argv[])

{

// Enable logging for UdpClient and UdpServer

LogComponentEnable("UdpClient", LOG_LEVEL_INFO);

LogComponentEnable("UdpServer", LOG_LEVEL_INFO);

bool useV6 = false;

Address serverAddress;

CommandLine cmd;

cmd.AddValue("useIpv6", "Use Ipv6", useV6);

cmd.Parse(argc, argv);

// Create the nodes required by the topology

NS_LOG_INFO("Create nodes.");

NodeContainer n;

n.Create(2);

InternetStackHelper internet;

internet.Install(n);

// Create the channels required by the topology

NS_LOG_INFO("Create channels.");

CsmaHelper csma;

csma.SetChannelAttribute("DataRate", DataRateValue(DataRate(5000000)));

csma.SetChannelAttribute("Delay", TimeValue(MilliSeconds(2)));

csma.SetDeviceAttribute("Mtu", UintegerValue(1400));

NetDeviceContainer d = csma.Install(n);

// Assign IP Addresses

NS_LOG_INFO("Assign IP Addresses.");

if (useV6 == false)

{

Ipv4AddressHelper ipv4;

ipv4.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer i = ipv4.Assign(d);

serverAddress = Address(i.GetAddress(1));

}

else

{

Ipv6AddressHelper ipv6;

ipv6.SetBase("2001:0000:f00d:cafe::", Ipv6Prefix(64));

Ipv6InterfaceContainer i6 = ipv6.Assign(d);

serverAddress = Address(i6.GetAddress(1, 1));

}

// Create Applications

NS_LOG_INFO("Create Applications.");

// UDP Server on node 1

uint16_t port = 4000;

UdpServerHelper server(port);

ApplicationContainer apps = server.Install(n.Get(1));

apps.Start(Seconds(1.0));

apps.Stop(Seconds(10.0));

// UDP Client on node 0

uint32_t MaxPacketSize = 1024;

Time interPacketInterval = Seconds(0.05);

uint32_t maxPacketCount = 320;

UdpClientHelper client(serverAddress, port);

client.SetAttribute("MaxPackets", UintegerValue(maxPacketCount));

client.SetAttribute("Interval", TimeValue(interPacketInterval));

client.SetAttribute("PacketSize", UintegerValue(MaxPacketSize));

apps = client.Install(n.Get(0));

apps.Start(Seconds(2.0));

apps.Stop(Seconds(10.0));

// Run the simulation

NS_LOG_INFO("Run Simulation.");

Simulator::Run();

Simulator::Destroy();

NS_LOG_INFO("Done.");

return 0;

}

OUTPUT:

Practical - 8

Aim: Program to simulate DHCP server and Clients.

dhcp77.cc

#include "ns3/core-module.h"

#include "ns3/internet-apps-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("DhcpExample");

int main(int argc, char *argv[])

{

CommandLine cmd;

bool verbose = true;

bool tracing = false;

cmd.AddValue("verbose", "Turn on the logs", verbose);

cmd.AddValue("tracing", "Turn on the tracing", tracing);

cmd.Parse(argc, argv);

// Enable log components

if (verbose)

{

LogComponentEnable("DhcpServer", LOG_LEVEL_ALL);

LogComponentEnable("DhcpClient", LOG_LEVEL_ALL);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

}

Time stopTime = Seconds(20);

// Create nodes

NS_LOG_INFO("Create nodes.");

NodeContainer nodes;

NodeContainer router;

nodes.Create(3);

router.Create(2);

NodeContainer net(nodes, router);

// Create CSMA channel

NS_LOG_INFO("Create channels.");

CsmaHelper csma;

csma.SetChannelAttribute("DataRate", StringValue("5Mbps"));

csma.SetChannelAttribute("Delay", StringValue("2ms"));

csma.SetDeviceAttribute("Mtu", UintegerValue(1500));

NetDeviceContainer devNet = csma.Install(net);

// Create Point-to-Point connection

NodeContainer p2pNodes;

p2pNodes.Add(net.Get(4)); // router node

p2pNodes.Create(1);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

NetDeviceContainer p2pDevices = pointToPoint.Install(p2pNodes);

// Install internet stack

InternetStackHelper tcpip;

tcpip.Install(nodes);

tcpip.Install(router);

tcpip.Install(p2pNodes.Get(1));

// Assign IP addresses for P2P link

Ipv4AddressHelper address;

address.SetBase("172.30.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign(p2pDevices);

// Static routing

Ipv4StaticRoutingHelper ipv4RoutingHelper;

Ptr<Ipv4> ipv4Ptr = p2pNodes.Get(1)->GetObject<Ipv4>();

Ptr<Ipv4StaticRouting> staticRoutingA =

ipv4RoutingHelper.GetStaticRouting(ipv4Ptr);

staticRoutingA->AddNetworkRouteTo(

Ipv4Address("172.30.0.0"),

Ipv4Mask("/24"),

Ipv4Address("172.30.1.1"),

1

);

// DHCP setup

NS_LOG_INFO("Setup DHCP applications.");

DhcpHelper dhcpHelper;

// Fixed IP for router

Ipv4InterfaceContainer fixedNodes =

dhcpHelper.InstallFixedAddress(

devNet.Get(4),

Ipv4Address("172.30.0.17"),

Ipv4Mask("/24")

);

// DHCP Server

ApplicationContainer dhcpServerApp =

dhcpHelper.InstallDhcpServer(

devNet.Get(3),

Ipv4Address("172.30.0.12"),

Ipv4Address("172.30.0.0"),

Ipv4Mask("/24"),

Ipv4Address("172.30.0.10"),

Ipv4Address("172.30.0.15"),

Ipv4Address("172.30.0.17")

);

DynamicCast<DhcpServer>(dhcpServerApp.Get(0))->AddStaticDhcpEntry(

devNet.Get(2)->GetAddress(),

Ipv4Address("172.30.0.14")

);

dhcpServerApp.Start(Seconds(0.0));

dhcpServerApp.Stop(stopTime);

// DHCP Clients

NetDeviceContainer dhcpClientNetDevs;

dhcpClientNetDevs.Add(devNet.Get(0));

dhcpClientNetDevs.Add(devNet.Get(1));

dhcpClientNetDevs.Add(devNet.Get(2));

ApplicationContainer dhcpClients =

dhcpHelper.InstallDhcpClient(dhcpClientNetDevs);

dhcpClients.Start(Seconds(1.0));

dhcpClients.Stop(stopTime);

// UDP Echo Server

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps =

echoServer.Install(p2pNodes.Get(1));

serverApps.Start(Seconds(0.0));

serverApps.Stop(stopTime);

// UDP Echo Client

UdpEchoClientHelper echoClient(

p2pInterfaces.GetAddress(1), 9

);

echoClient.SetAttribute("MaxPackets", UintegerValue(100));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps =

echoClient.Install(nodes.Get(1));

clientApps.Start(Seconds(10.0));

clientApps.Stop(stopTime);

// Enable tracing

if (tracing)

{

csma.EnablePcapAll("dhcp-csma");

pointToPoint.EnablePcapAll("dhcp-p2p");

}

// Run simulation

Simulator::Stop(stopTime + Seconds(10.0));

NS_LOG_INFO("Run Simulation.");

Simulator::Run();

Simulator::Destroy();

NS_LOG_INFO("Done.");

return 0;

}

OUTPUT:

Using Python Visualizer

Practical - 9

Aim: Program to simulate FTP using TCP

ftp77.cc

#include <string>

#include <fstream>

#include "ns3/core-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/internet-module.h"

#include "ns3/applications-module.h"

#include "ns3/network-module.h"

#include "ns3/packet-sink.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("TcpBulkSendExample");

int main(int argc, char *argv[])

{

bool tracing = false;

uint32_t maxBytes = 0;

// Command-line arguments

CommandLine cmd;

cmd.AddValue("tracing", "Flag to enable/disable tracing", tracing);

cmd.AddValue("maxBytes", "Total number of bytes for application to send", maxBytes);

cmd.Parse(argc, argv);

// Create nodes

NS_LOG_INFO("Create nodes.");

NodeContainer nodes;

nodes.Create(2);

// Create point-to-point channel

NS_LOG_INFO("Create channels.");

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("500Kbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("5ms"));

NetDeviceContainer devices;

devices = pointToPoint.Install(nodes);

// Install internet stack

InternetStackHelper internet;

internet.Install(nodes);

// Assign IP addresses

NS_LOG_INFO("Assign IP Addresses.");

Ipv4AddressHelper ipv4;

ipv4.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer i = ipv4.Assign(devices);

// Create Applications

NS_LOG_INFO("Create Applications.");

// TCP Bulk Send on node 0

uint16_t port = 9; // well-known port

BulkSendHelper source(

"ns3::TcpSocketFactory",

InetSocketAddress(i.GetAddress(1), port)

);

// Set total bytes to send (0 = unlimited)

source.SetAttribute("MaxBytes", UintegerValue(maxBytes));

ApplicationContainer sourceApps = source.Install(nodes.Get(0));

sourceApps.Start(Seconds(0.0));

sourceApps.Stop(Seconds(10.0));

// Packet Sink on node 1

PacketSinkHelper sink(

"ns3::TcpSocketFactory",

InetSocketAddress(Ipv4Address::GetAny(), port)

);

ApplicationContainer sinkApps = sink.Install(nodes.Get(1));

sinkApps.Start(Seconds(0.0));

sinkApps.Stop(Seconds(10.0));

// Enable tracing if required

if (tracing)

{

AsciiTraceHelper ascii;

pointToPoint.EnableAsciiAll(ascii.CreateFileStream("tcp-bulk-send.tr"));

pointToPoint.EnablePcapAll("tcp-bulk-send", false);

}

// Run simulation

NS_LOG_INFO("Run Simulation.");

Simulator::Stop(Seconds(10.0));

Simulator::Run();

Simulator::Destroy();

NS_LOG_INFO("Done.");

// Output total bytes received

Ptr<PacketSink> sink1 = DynamicCast<PacketSink>(sinkApps.Get(0));

std::cout << "Total Bytes Received: " << sink1->GetTotalRx() << std::endl;

return 0;

}

Output:

Using Python Visualizer

Practical – 10

Aim: Exercises for analyzing the network protocols using Wireshark

• Capture the packets while browsing the any web site

• Analyze the header fields of various protocols

- WINDOWS

- ip.len le 1500

- tcp.window_size != 0 && tcp.flags.reset != 1

- tcp

- udp

- ipv6

- frame.time != "Sep 26, 2026 23:18:04.954975"

- IP

- ARP

- ip.addr != 10.43.54.65

- ip.src != 10.43.54.65 or ip.dst != 10.43.54.65

- LINUX

Analyzing Pcap files in point to point network using Wireshark